NMD 100

Note and Media Dispenser

Product Datasheet & Integration Reference

Modular • Scalable • High-Reliability ATM Banknote Dispenser

Product Overview

The NMD 100 (Notes and Media Dispenser) is a high-performance, modular banknote dispensing system developed by De La Rue for ATM integration. Designed for reliability and flexibility, the NMD 100 supports 1 to 8 denominations in a single chassis and can be configured for front or rear service access. Its stackable denomination extension (DE) architecture makes field upgrades simple without major cabinet modifications.

Key Features

- Supports 1 to 8 Note Cassettes — handles up to 8 different denominations simultaneously

- Modular, stackable architecture — field-upgradable with Denomination Extension (DE) kits

- Front service and rear service configurations available

- High-capacity cassettes with secure locking mechanism

- Integrated Reject Vault for sorted note rejection

- Note Qualifier (NQ) module for accurate banknote authentication

- Central Machine Controller (CMC 200) with RS-232C communication interface

- Parallelism-guided installation for precise cabinet alignment

- Compatible with most ATM cabinet formats via slide mounting system

- ESD protection on all internal inter-module connections

Product Gallery

| Front View — Cassette Side | Rear View — Service Side |
| --- | --- |

| Internal Transport Mechanism | Multi-Cassette Configuration (up to 8DN) |
| --- | --- |

Technical Specifications

| Weight & Physical Dimensions |  |
| --- | --- |
| Standard 2DN NMD (with empty cassettes & RV) | 22.7 kg |
| Additional denomination (DE kit) | 2.6 kg per unit |
| Note Cassette (without notes) | 2.9 kg |
| Reject Vault (without notes) | 2.0 kg |
| NMD Base Width | 366.6 ± 1 mm |
| NMD Height (2DN configuration) | Approx. 449.5 mm (85 mm ext.) |
| NMD Depth | Approx. 240 mm |
| Delivery Slot Width | 144 ± 0.2 mm |

| Power Supply |  |
| --- | --- |
| Mains Input Voltage | 120 – 240 VAC (nominal) |
| Input Current | 0.8 – 2.0 A |
| Frequency | 50 – 60 Hz |
| Mains Connector | IEC-320 Type T65 |
| DC Distribution | From integrated Power Supply to all modules |

| Communication Interface |  |
| --- | --- |
| Standard | RS-232C |
| Connector (NMD side) | CT 5-pin, AMP 173 977-5 |
| Connector (Host side) | D-SUB 9-pin, AMP 748 047-1 (shielded) |
| Cable | AWG26 × 6, shielded |
| Protocol | De La Rue CCRM NMD 100 (Doc. 8030676) |
| Handshaking | With or without hardware handshaking |

| Shutter Control Interface |  |
| --- | --- |
| Connector Type | AMP CT, P/N 176125-8 |
| Reference Document | DLR Document D003242 |
| Location | CMC 200 board |
| Cable (if own shutter) | AMP 173 977-8, AWG 26 recommended |

| Installation & Mounting |  |
| --- | --- |
| Service Access Modes | Front service or rear service |
| Mounting Method | Side-mounting with parallel slides (pull-out capable) |
| Parallelism Requirement | Max. 1.0 mm difference between mounting points at same axis |
| Horizontal tolerance | ± 50 mm |
| Min. front clearance (refill) | 500 mm |
| Cassette Configuration | 1 to 8 Note Cassettes (up to 8 denominations) |
| Extension Method | DE (Denomination Extension) kits — field upgradable |

Door Control PCB — ATK DOOR-V1.1

The ATK DOOR-V1.1 is the controller board for the dispensing door mechanism. It interfaces with the host via RS-232 and manages motor control, sensor inputs, and power distribution.

| ATK DOOR-V1.1 PCB | ATK DOOR-V1.1 — I/O Port Annotations |
| --- | --- |

| ATK DOOR-V1.1 Interface Specification |  |
| --- | --- |
| Power Input | 12 V DC |
| Power Outputs | 12 V output × 1, 5 V output × 2 |
| Motor Control | 12 V motor forward/reverse (H-bridge) |
| Sensor Inputs | 6 × sensor receiver ports (6-position detection) |
| Communication | RS-232 interface (DB-9 connector) |
| Board Dimensions | Approx. 100 × 85 mm |
| PCB Version | ATK DOOR-V1.1 |

Roll Shutter Mechanism — 17010

The 17010 roll shutter provides the dispensing aperture security mechanism, integrating with the ATK DOOR-V1.1 controller board.

| Roll Shutter Assembly (17010) | Roll Shutter Dimensional Drawing |
| --- | --- |

| Roll Shutter Dimensional Specification |  |
| --- | --- |
| Overall Width | 351 mm |
| Body Width | 337 mm |
| Overall Height | 148.5 mm |
| Aperture Height | 112 mm |
| Depth (side view) | 55 mm |
| Side Profile Height | 115 mm |
| Mounting Offset (bottom) | 9.5 mm |
| Slot Width | 8 mm |
| Slot Offset | 9.5 mm |
| Drive Gear | Left side, motor-driven via ATK DOOR-V1.1 |

System Architecture

The NMD 100 is composed of modular sub-assemblies that can be stacked and combined for flexible denominations. The following diagram describes the module hierarchy.

| NMD 100 Module Hierarchy |  |
| --- | --- |
| Stack Presenter (SPF/SPR) | Presents stacked notes to the customer via delivery slot |
| Bundle Carriage Unit (BCU) | Transports bundled notes from sorter to presenter |
| Bundle Outlet Unit (BOU) | Manages exit path to delivery chute |
| Note Stacker (NS) | Accumulates notes for batch dispensing |
| Note Qualifier (NQ) | Authenticates and validates notes during transport |
| Note Feeder (NF) | Picks individual notes from the cassette |
| Frame (FR) | Main structural chassis housing all modules |
| Central Machine Controller (CMC 200) | Main control board — RS-232 host interface, power management |
| Reject Vault (RV) | Secure lockable container for rejected notes |
| Note Diverter (ND) | Routes notes between denomination paths |
| Denomination Extension (DE) | Adds additional cassette support — field installable |
| Note Cassette (NC) | Lockable removable cassette for each denomination |
| Note Feed Controller (NFC) | Sub-controller per denomination layer |

Configuration Options

| Cassette Options |  |
| --- | --- |
| Minimum Configuration | 1 × Note Cassette + 1 × Reject Vault |
| Standard Configuration | 2–4 denominations |
| Maximum Configuration | 8 denominations (via DE kits) |
| Note Cassette Capacity | Varies by model (see NC product specifications) |
| Cassette Security | Customer-selectable lock coding |
| Service Configuration |  |
| Rear Service | Default — cassettes and service from rear of cabinet |
| Front Service | Optional — cassettes accessible from ATM front |
| Extension Kits |  |
| DE Kit (1 addition) | Adds 1 denomination — includes NF, NFC, DE frame, NC |
| Note Cassette Coding | Refer to Service Manual 8030666 |

Compliance & Safety

| Safety & Compliance |  |
| --- | --- |
| ESD Protection | All inter-module and signal connections require ESD precautions during service |
| EMC Responsibility | System integrator responsible for EMC compliance |
| Cabinet Transport | NMD must be transported upright, cassettes removed, properly secured |
| Filter Connector | Recommended when RS-232 cable exits the cabinet |
| Reference Standards | Refer to De La Rue product specifications D018140 for full environmental specs |

Reference Documents

| Related Documentation |  |
| --- | --- |
| NMD 100 Integration Manual | Doc. No. 8030614/6-E (De La Rue) |
| NMD 100 Product Specifications | Doc. No. D018140 (De La Rue) |
| NMD 100 Getting Started Guide | Doc. No. 8030685 (De La Rue) |
| NMD 100 CCRM | Doc. No. 8030676 — Communication & Command Reference |
| NMD 100 Service Manual | Doc. No. 8030666 — Cassette Coding & Maintenance |
| Shutter Control Interface | DLR Document D003242 |
| CashCode MSM Spec | Shenzhen Zhongkong, Material No. 13540040, V1.0 |

NMD 100 Product Datasheet  |  De La Rue Cash Systems  |  Talja Södra Industriområde, SE-642 84 Flen, Sweden

This document is compiled from official integration references and supplier specifications. For full technical data refer to De La Rue documents listed above.